Cell size and Cluster

Determine the number of Full-Duplex Channels available in a cluster and the total capacity of a cellular system where there are 10 clusters, each consisting of 20 cells with 18 channels in each.

Ans: Total number of Full-Duplex channel in cluster = 20 x 18 = 360

Link Budget Design

Evaluate the feasibility of a 5 km link, with one access point and one client radio. The access point is connected to an antenna with 10dBi gain, with a transmitting power of 20 dBm and a receive sensitivity of -89dBm. The client is connected to an antenna with 14dBi gain, with a transmitting power of 15dBm and a receive sensitivity of -82dBm. The cables in both systems are short, with a loss of 2dB at each side at the 2.4 GHz frequency of operation.

Solution:

The feasibility of a 5 km link has one access point and one client radio.

The access point is connected to an antenna with 10 dBi gain, with a transmitting power of 20 dBm and a receive sensitivity of -89 dBm.

The client is connected to an antenna with 14 dBi gain, with a transmitting power of 15 dBm and a r eceive sensitivity of -82 dBm.

The cables in both systems are short, with a loss of 2dB at each side at the 2.4 GHz frequency of operation.

Free space Path loss (Lf)=10log10λ2(4π)2d2=-114dB

Path loss @ 5km link = -114dB

Modulation Techniques and Channel Decoding.

Write the waveforms to explain MSK transmission for a binary sequence 1010101.

Draw the waveforms to explain QPSK transmission for a binary sequence 1011001 modulated under QPSK techniques.

Why convolutional coding is memory coding, explain with an example where input stream is i = 10110….

Describe convolutional encoder with code dimension (2,1,3) and input message 10011 with generator polynomial  g1x=x2+x+1 and g2x=x2+1. Also determine the final codeword.

For a given (6,3) linear block code, the generator matrix G is given as

G = 100010001    101011110

- All corresponding code vectors

- Minimum Hamming Distance

- Verify that code is a single error correcting code

- Parity check matrix

- Determine the transmitted codeword if received codeword is 100011

Find the linear block code encoder G if the generator polynomial g(x) = 1+x2+x3 for a (7,4) code.

Find the generator polynomial g(x) for a (7,4) cycle code and find the code vectors for the following data vectors:1010 and 1011

The generator polynomials for a 1/3 binary convolutional encoder is given by:

g1x=x3+x2+1

g2x=x3+x2

g3x=x3+1

Then calculate the encoded binary sequence for input sequence 011.

A spread spectrum communication system is characterized by the following parameters.

Duration of each information bit, Tb=4.095ms

Chip duration of a PN Sequence, Tc=1μs

Determine the processing gain and jamming margin if EbN0 =10 and the average probability of error Pe=0.5 ×10-5.

Evaluate the feasibility of a 5 km link, with one access point and one client radio. The access point is connected to an antenna with 10dBi gain, with a transmitting power of 20 dBm and a receive sensitivity of -89dBm. The client is connected to an antenna with I4dBi gain, with a transmitting power of 15dBm and a receive sensitivity of -82dBm. The cables in both systems are short, with a loss of 2dB at each side at the 2.4 GHz frequency of operation.