Let X be the code vector such that the elements of code words are :

X= x0, x1, x2, x3, ……………., xn-1

We know that each of these codewords above consists of message bits and parity bits.

That is,

x0= [ m0:c0] where m0 means message bits and c0 means parity bits

x1= [ m1:c1]

……

……

xi= [ mi:ci-k] = mi for i=0, 1, ……, k-1

= ci-k for i = k, k+1, ……, n-1

And thus,

X= M:C ………………………… (i)

Where,

M = k – message vectors

C = n-k -> parity vectors

Encoding at the sender to form codeword from given message bits

Now at sender side, we pass down the message bits k to the channel encoder which uses generator matrix to generate a codeword(parity appended to message bits) using block of message bits,

The codeword X can be generated using message M as

X = M.G………………………… (ii)

Considering the k bit message, n bit codeword, n-k bit parity word,

In matrix form,

X1 × n=M1 × k .Gk × n………………………… (iii)

Where,

X1 × n=

M1 × k=

Gk × n=

Generator Matrix, Gk × n

This generator matrix is generally represented as,

G=IK | P

Where,

IK= k × k identity matrix

P = k ×(n-k) coefficient matrix that can have 1 or 0 as its value.

Therefore,

IK=100…..00001k×k

P=P00P10 0..P00P11P0(k-1)Pn-k-100Pn-k-1(n-k)k×(n-k) [Note: here, size is column x row]

Now,

From eqn.(i) , eq(ii) and eq(iii)

X=  M:C  = [M].[G] =[M] IK | P

Or,  M:C = [M. IK : M.P ]

The parity vector,

C = M.P

In vector form,

[C0, C1,………………………, Cn-k]1×n-k=

[M0, M1,……, Mk-1]1×k-1 × P00P10 0..P01P11P0(k-1)Pn-k-100Pn-k-1(k-1)k×(n-k)

Thus,

C0=M0× P00 ⨁ M1 × P01 ⨁……………..⨁ Mk-1 × P0(k-1)

C1=M0× P10 ⨁ M1 × P11 ⨁……………..⨁ Mk-1 × P1(k-1)

…….

Cn-k=M0× P(n-k-1)0 ⨁ M1 × P(n-k-1)1 ⨁……………..⨁ M(n-k-1) × P1(k-1)

In the above equation, the addition is modulo – 2 additions.

Questions:

The generator matrix for a (6,3) block code is given below. Find all the code vectors.

G = 100010001    101011110

- All corresponding code vectors

- Minimum Hamming Distance

- Verify that code is a single error correcting code

- Parity check matrix

- Determine the transmitted codeword if received codeword is 100011

Solution:

(n, k) = (6, 3) block code

i.e. n = number of bits in codeword = 6 bits

k = number of bits in message word (data word) = 3 bits

n – k = number of parity bits = 6 – 3 = 3 bits

Now, Generator matrix, Gk × n= IK | P

Where

IK= k × k identity matrix

P = k ×(n-k) coefficient matrix that can have 1 or 0 as its value

From given generator matrix,

I3=[  ]

P3×3=10 1011110k×(n-k)

For k = 3 bits message

There are 8 possible combinations for message M

M0 M1 M2=0 0 0

M0 M1 M2=0 0 1

M0 M1 M2=0 1 0

M0 M1 M2=0 1 1

M0 M1 M2=1 0 0

M0 M1 M2=1 0 1

M0 M1 M2=1 1 0

M0 M1 M2=1 1 1

Then, Parity bits C

C = M.P

For, m0=M0 M1 M2=0 0 0

c0=[M0, M1,M2]1×3    10 10111103×3

=[0 0 0]1×3    10 10111103×3

=[0 0 0]

Codeword, x0=c0:m0=[0 0 0 0 0 0]

For, m1=M0 M1 M2=0 0 1

c1=[M0, M1,M2]1×3    10 10111103×3

=[0 0 1]1×3    10 10111103×3

=[1 1 0]

Codeword, x1=c1:m1=[0 0 1 1 1 0]

Similarly,

For, m2=M0 M1 M2=0 1 0

Codeword, x2=c2:m2=[0 1 0 0 1 1]

For, m3=M0 M1 M2=0 1 1

Codeword, x3=c3:m3=[0 1 1 1 0 1]

For, m4=M0 M1 M2=100

Codeword, x4=c4:m4=[1 0 0 1 0 1]

For, m5=M0 M1 M2=1 0 1

Codeword, x5=c5:m5=[1 0 1 0 1 1]

For, m6=M0 M1 M2=1 1 0

Codeword, x6=c6:m6=[1 1 0 1 1 0]

For, m7=M0 M1 M2=1 1 1

Codeword, x7=c7:m7=[1 1 1 0 0 0]

Finally, the collection of all these codewords is required code vectors

X= x0, x1, x2, x3, x4, x5, x6, x7

X=0 0 0 0 0 0 ,0 0 1 1 1 0,…………………………….,1 1 1 0 0 0

Minimum Hamming Distance and Hamming weight

Hamming weight: Number of ones in the non-zero codeword.

For e.g. Codeword, x7=c7:m7=[1 1 1 0 0 0]

Hamming weight: 3

Minimum Hamming Distance: Smallest hamming weight in the non-zero codeword

For our question, minimum number of one in non-zero codeword is 3.

Therefore dmin=3

Correction and detection of the bits based on minimum hamming weight in linear block code:

Condition to detect up to “s” errors per codeword

dmin≥s+1

s≤dmin-1

For above question,

s=dmin-1

s=3-1

s=2

We can detect 2 errors per codeword.

Condition to detect and correct up to “t” errors per codeword

dmin≥2t+1

=dmin-12

For above question,

t= 1

We can detect and correct up to 1 error per codeword.

Condition to detect up to “s” errors and correct up to “t” errors per codeword

dmin≥t+s+1

Conclusion:

At sender side, we have:

- Encoding

- Encoding by: Generator Matrix

- Formation of Codewords (Message bits + Parity Bits) by Generator Matrix

At receiver side we have opposite operation

- Decoding

- Decoding by: Parity Check Matrix H (Which is given by Generator Matrix)

- Output the result message bits

Example:

For Our Question of (6,3) linear block code,

Generator Matrix, G at sender side

G = 100010001    101011110

Ik	      P

Then Parity Check Matrix, H at receiver side

H = 101011110    100010001

PT	        In-k

Here,

PT = transpose of coefficient matrix [P]

In-k= Identity matrix of size (n-k)(n-k)

How to confirm that the received codeword is error free at the receiver?

- Find the Transpose of the Parity Check matrix H

HT=101011110100010001

- Apply X.HT

Case 1: No error

Suppose we have sent codeword X from sender and received codeword R = X at the receiver,

X.HT = 0, then the received codeword is error free.

Case 2: Error

Suppose we have sent codeword X from sender and

received codeword (R) =X+E at the receiver, then

R.HT = X.HT+E.HT

= E.HT

This term R.HT is known as syndrome and denoted by (S)

S=R.HT

This process of finding error or the correct codeword is known as syndrome calculation.

Qn. Determine the transmitted codeword if received codeword is 100011.

Soln,

Given

R = [100011]

S = R.HT=[1 0 0 0 1 1] 101011110100010001

=[1.1⨁0.0⨁0.1⨁0.1⨁1.0⨁1.0    1.0⨁0.1⨁0.1⨁0.0⨁1.1⨁1.0    1.1⨁0.1⨁0.0⨁0.0⨁1.0⨁1.1]

=[1 1 0]

Since, S=R.HT≠0

There is an error in the codeword received.

This syndrome matches 3rd row of the HT, Error in bit 3 of received codeword [1 0 0 0 1 1]

Ans: Transmitted actual codeword is [1 0 1 0 1 1]

Qn. Determine the transmitted codeword if received codeword is 100101.

Soln,

Given

R = [100011]

S = R.HT=[1 0 0 1 0 1] 101011110100010001

=[0 0 0]

Since, S=R.HT=0,

There is no error in the codeword received.

Transmitted codeword = Received codeword = 1

TYPE 2 Question:

Question given from Receiver side,

An error control code has the following parity check matrix

H = 101110011    100010001

- Determine the generator matrix.

- Find the codeword for message [1 0 1]

- Decode received codeword 100110

Soln,

We know

H = PT | In-k

From question,

H = 101111010    100010001

Then,

PT=101110011

I3=100010001

Since, Generator Matrix Gk × n= IK | P ,

We have to find,

P = (PT)T =110011101

Ik=I3=100010001

So,

- Generator Matrix Gk × n= IK | P =100010001    110011101

To find codeword, we need to find the parity C, given by

C = MP

- Codeword, x0 = [M:C]

To decode received codeword, we use syndrome calculation

S = R.HT=[1 0 0 1 1 0] 101011110100010001

S = [1 0 1] ≠0, there is error in the codeword received.

This syndrome matches 1st row of the HT, Error in bit 1 of received codeword [1 0 0 1 1 0]

- Transmitted codeword = [0 0 0 1 1 0]

Binary Cyclic Code